# Report: LiPo Battery Monitor and Alarm System

Group Members: Ryan Bezerra, Ava Marshia, Alis Craig ​

Date: April 19, 2026

Subject: Electric and Hybrid Vehicles

Table of Contents

- Introduction

- Materials and Components

- Methods: How it Works

  - Voltage Sensing and Scaling

  - Calculating Individual Cell Values

  - Alarm Trigger Logic

- Research and Results

- Conclusion

# Introduction

Lithium Polymer (LiPo) batteries are common in hobby electronics and robotics because they can hold a lot of power. However, they are sensitive. If a single cell's voltage drops too low, the battery can be permanently damaged or even become a fire hazard. This report looks at how a standard 1-8S battery monitor works to prevent these issues by checking voltages and sounding an alarm when needed.

Additionally, we will go over why we chose to go with a commercially available monitor rather than creating our own from scratch.

# Materials and Components

We made use of a single commercially available LiPo battery monitor, but for the sake of explanation, the make up of the device is provided below. Additionally, the device we made use of can be found here.

The device is a compact piece of hardware that plugs into a battery's balance lead. The main parts used in its construction include:

- Microcontroller (MCU): The main chip that handles all the math and controls the screen and buzzers.

- 7-Segment LED Display: A 3-digit screen that shows the voltage numbers.

- Voltage Dividers: A set of resistors that lower the high battery voltage so the MCU can read it safely.

- Piezo Buzzers: Two loud speakers that make the alarm sound.

- Analog-to-Digital Converter (ADC): A tool inside the MCU that changes the electrical signal into a digital number the chip can understand.

# Methods: How it Works

## Voltage Sensing and Scaling

Microcontrollers usually run on 3.3V or 5V. If you tried to plug a 22V battery directly into the chip, it would burn out instantly. To fix this, the device uses voltage dividers. These are pairs of resistors that "step down" the voltage. For example, if the battery is at 20V, the resistors might scale it down to exactly 2V. The MCU reads that 2V and knows it actually represents 20V based on the resistor values.

## Calculating Individual Cell Values

The balance lead of a battery has a wire for every cell. The monitor checks each wire's voltage relative to the ground wire. Since the cells are wired in a series, the MCU has to do some subtraction to find the voltage of just one cell. For example, to find the voltage of the second cell, the MCU subtracts the first wire's reading from the second wire's reading.

## Alarm Trigger Logic

The alarm is controlled by code. The user can usually press a button on the back to set a "floor" voltage (like 3.3V). The MCU constantly checks every cell. If it sees any single cell go below that floor, it turns on a transistor that sends power to the buzzers. It keeps beeping until the voltage goes back up or the battery is unplugged.

# Research and Results

In our look at this device, we found that accuracy is very important. Because the device uses resistors to scale the voltage, the quality of those resistors matters. If the resistors are even 1% off, the voltage reading on the screen might be wrong by 0.1V or more. Most of these monitors are accurate enough for general use, but they are best used as a safety backup rather than a professional lab tool.

We also noticed that the alarm is extremely loud (around 100 decibels). This is necessary because these batteries are often used in loud environments, like near racing drones or RC planes, where a quiet beep wouldn't be heard. On top of the fact that an being notified about a LiPo batteries status is very important for the health of the battery and to avoid any unnecessary hazards.

We chose to use a commercially available device for the coincidental reason that commercially available devices already have these crucial elements nailed down, and we wouldn’t have to do weeks of testing to get it right on our own, on top of the fact that these devices have already been made so cheap that financially it wouldn’t have made sense to spend extra to do the work ourselves.

# Conclusion

The 1-8S LiPo monitor is a simple but effective piece of engineering. By combining basic resistor math with a microcontroller, it provides a vital safety net for expensive batteries. It is an essential tool for anyone working with multi-cell LiPo packs, as it provides both visual data and an audible warning that can prevent battery failure.